CONFINED SPACES

What is a Confined Space?

- Any space of an enclosed nature where there is a risk of death or injury from hazardous substances or dangerous conditions (e.g. lack of oxygen)

- Limited ways in / out

- Contains hazardous substances or atmosphere

- Risk of being trapped / engulfed / drowned

Examples include grain silos, wood pellet stores, sewers, areas with intentionally reduced oxygen such as fruit stores, and storage tanks.

Confined spaces are not designed to be worked in for long periods. Avoid entering a confined space unless you are trained and equipped to do so and it is absolutely necessary.

Hazards

- Lack of oxygen (suffocation)

- Fire / explosion risk

- Build up of toxic substances

- Physical hazards, e.g. noise, extreme temperatures, mechanical hazards, falls, electrocution, machinery

Before entering a confined space, you must:

- Ensure a risk assessment has been carried out

- Have received specific training

- Ensure atmospheric testing has been carried out

- Isolate the space

- Turn off gas, power, etc. and ventilate for at least 15 minutes

- Ensure all flammable / toxic materials or substances have been safely removed

- Ensure you understand all precautions to be taken

- Ensure at least three people are present (buddy system) unless rescue is possible with an external winch and the person in the confined space is permanently attached to a rescue line connected to the winch when two is acceptable

- Ensure you are medically fit

- Ensure permit to work is in place and signed, authorising the type of work

Permit to work

Do you understand the conditions of the permit, including:

- Who is allowed in?

- What barriers, warning signs, covers, etc. are needed?

- What personal protective equipment (PPE) must be worn?

- Whether ventilation and / or atmospheric monitoring is needed?

- What the rescue / emergency procedures are?

- What means of communication there are?

- What safety / monitoring equipment you need, e.g. gas monitor, safety harness, alarm and testing arrangements?

- How long it lasts?

- Any special instructions or precautions?

Legislation and Resources

- The Health and Safety at Work etc Act 1974

- The Confined Spaces Regulations 1997 apply where there is a potential risk from work in a confined space

- The Management of Health and Safety at Work Regulations 1999 require employers and self-employed people to carry out a suitable and sufficient assessment of the risks for work in confined spaces

- INDG258: Safe Working in Confined spaces

- The Personal Protective Equipment at Work Regulations 1992 (as amended)

REGISTER OF ATTENDANCE

| TOPIC COVERED | CONFINED SPACES |
| --- | --- |

| NAMES OF THOSE WHO ATTENDED THIS TOOLBOX TALK |  |
| --- | --- |
| Name (print) | Signature |
| Carried out by Name (print) | Carried out by Signature |
| Site | Date: |