IMSF 100 - Confined Spaces Checklist

This checklist should be used to verify that the safety of a specific confined space is adequate and is being maintained. The checklist should be used as part of the risk assessment.  It may also be used to support periodic inspections and audits, and in incident investigations.

An ‘No’ entered in a box will require justification and should be reflected in the risk assessment and an explanatory note added to the comments column.

Where appropriate, actions should be taken to address any control measures that are absent.

| Control Measure | Yes / No / NA | Comments |
| --- | --- | --- |
| 1. Eliminate / Avoid |  |  |
| Can the need to enter the confined space be avoided? (Consider provision of remote controls, video camera, inspection hatches) |  |  |
| Can the confined space be permanently re-classified by removing the enclosed nature? |  |  |
| Can the confined space be permanently re-classified by removing the specified hazard(s) that make it a confined space? (See Section 3 ). |  |  |
| Can the confined space be temporarily re-classified by the introduction of forced ventilation? (See Section 5 of this standard). (Note: the forced ventilation shall be at such a rate that any release of gas or fumes would be impossible to create an unsafe atmosphere). |  |  |
| In the event of forced ventilation failure, is there sufficient time for a safe evacuation to be achieved? |  |  |
| 2. Risk Assessment and Procedures |  |  |
| Has a specific risk assessment been completed for the confined space? |  |  |
| Has the risk assessment considered all conditions of operation, including abnormal operating conditions and temporary conditions, such as welding? |  |  |
| Has the risk assessment considered the risks which may be created by the planned work? (e.g., burning / welding / static discharges) |  |  |
| Has a specific safe working procedure (or ‘safe system of work’) been prepared, and is it relevant to the planned activity? |  |  |
| Does the safe working procedure specify the measures to be taken in the event of an emergency? |  |  |
| Has a Confined Space Permit be raised, including any control measures specified in this checklist? (Note: this is a mandatory requirement for confined spaces). |  |  |
| 3. Engineered controls |  |  |
| Has a suitable warning sign been posted to identify the confined space? |  |  |
| Has the confined space been secured (e.g. with a barrier, guard or gate) to deter unauthorised access? |  |  |
| Have fixed gas detectors or low oxygen monitors been provided, and are they in a suitable location? |  |  |
| Are gas detectors adequately maintained and calibrated? |  |  |
| Can the confined space be ventilated naturally to reduce the risk of an unsafe atmosphere? (e.g., by opening or removing doors or hatches) |  |  |
| Have fixed ventilation provisions been made? |  |  |
| Is fixed ventilation maintained and tested periodically? |  |  |
| Have cameras been fitted if identified as a control so that persons inside the confined space can be monitored? |  |  |
| Is there an adequate means of communication between persons inside the confined space and those outside? |  |  |
| 4. Ambient conditions |  |  |
| Is sufficient ventilation available? (Note, oxygen should not be forced into a confined space as this can create an explosive atmosphere) |  |  |
| Is the level of lighting inside the confined space adequate? |  |  |
| Can the ambient temperature in the confined space be controlled / monitored to prevent excessive build-up in temperature of the operatives? |  |  |
| 5. Access Requirements |  |  |
| Has adequate access been provided for safe entry into the confined space? |  |  |
| Can the site ensure that any walkways and access platforms to the opening of the confined space are safe to use? |  |  |
| Is the size and configuration of access sufficient to cater for people wearing PPE, including emergency breathing apparatus (BA) in case of an emergency? (Note: the minimum permissible size for emergencies involving BA is 575mm square Minimum for all other activities – Round Hole 457mm diameter, Rectangular Hole – 457mm x 410mm). |  |  |
| Is sufficient access equipment, such as a tripod, full body harness and winch available for access/egress into the confined space? |  |  |
| 6. Training, and competence and awareness |  |  |
| Have all persons who are required to manage / enter a confined space received specific training, demonstrate competency and authorised before commencing work? |  |  |
| Where the risk assessment has highlighted physical constraints as a result of the layout or task, are the individuals of suitable build, and fitness to complete the task? |  |  |
| Has the site appointed sufficient competent persons to issue a “permit to work” and supervise the activities being undertaken within the confined space? |  |  |
| Are all employees/contractors aware that a Permit to Work must be issued prior to any person entering a confined space? |  |  |
| Are all employees/contractors aware of the procedures when changes to the task, the working environment or the method of work changes? |  |  |
| 7. Pre-entry safeguards |  |  |
| Has the confined space been emptied of its contents, including residues, where possible? |  |  |
| Has the inside of the confined space been cleaned? (Note that cleaning methods may require inclusion in the specific risk assessment). |  |  |
| Can pipe work be isolated to prevent gas, fume or vapour entering the confined space? |  |  |
| Can mechanical and electrical isolation of equipment be completed, to prevent inadvertently operating or discharging into the confined space? |  |  |
| Are all isolation checked before work commences as part of the Permit to Work? |  |  |
| Has the atmosphere in the confined space been verified as safe, prior to entry? |  |  |
| Are signs displayed on the outside of the confined space notifying that work is being undertaken and that people are inside? |  |  |
| 8. Procedural controls during entry |  |  |
| Where flammable or potentially explosive atmospheres are possible, are sources of potential ignition removed/controlled and included in the safe working procedures? |  |  |
| If appropriate, have non-sparking tools been specified and provided? |  |  |
| Are suitable precautions in place to prevent electric shock inside metal tanks? (e.g. use of low voltage equipment / residual current devices) |  |  |
| Have personal air monitoring devices (with current calibration certificate) been provided for all persons who will enter the confined space? (This will confirm that the oxygen levels are acceptable, and may also warn of the presence of flammable or toxic gases, depending on the model) |  |  |
| Have monitors and ELSA if absolutely necessary been checked prior to entry? |  |  |
| 9. Personal Protective Equipment |  |  |
| Has the correct type of PPE been specified for each activity? (This should include provision of rescue equipment such as harnesses/lifelines). |  |  |
| Has suitable PPE been provided for all persons entering a confined space, and is it being used? |  |  |
| Is PPE in good condition and is it appropriate for the conditions and working activities? |  |  |
| 10. Emergency Arrangements |  |  |
| Has an emergency plan been prepared for this specific confined space? |  |  |
| Has the site identified suitable emergency arrangements for the activity before the work commences? |  |  |
| Has a rescue procedure been defined and communicated? |  |  |
| Is there a suitable method of raising the alarm? |  |  |
| Has the response time of emergency services been considered? |  |  |
| Has necessary rescue and resuscitation equipment been identified and provided? |  |  |
| Is the first aid provisions adequate? |  |  |
| Are persons responsible for emergency response adequately trained and practised? |  |  |
| If breathing apparatus is required is the access a minimum of 575mm square? |  |  |